SANTHOSH D

192412209

ECA5605- 5G/6G-COMMUNICATION

EXP 3

Program

clear;

clc;

close all;

%% Representative bands and link-budget assumptions

freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz";

"Mid-band";

"mmWave"];

txPowerdBm = 30;

gtDbi = 0;

grDbi = 0;

distanceKm = logspace(log10(0.1), log10(2), 200);

distanceAtPointKm = 0.5;

%% Free-Space Path Loss (FSPL)

% d in km, f in MHz

fspl = @(fMHz, dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

lossAtPoint = fspl(freqMHz, distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

%% Link budget table

linkTable = table( ...

bandName, ...

freqGHz(:), ...

lossAtPoint(:), ...

rxAtPoint(:), ...

'VariableNames', ...

{'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'});

disp('Link Budget at 0.5 km');

disp(linkTable);

%% Path loss and received power versus distance

figure;

tiledlayout(1,2);

% Path loss plot

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

plot(distanceKm, ...

fspl(freqMHz(k), distanceKm), ...

'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Path Loss (dB)');

legend(bandName, 'Location', 'northwest');

title('Free-Space Path Loss');

% Received power plot

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

rx = txPowerdBm + gtDbi + grDbi - ...

fspl(freqMHz(k), distanceKm);

plot(distanceKm, rx, 'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Estimated Received Power (dBm)');

legend(bandName, 'Location', 'southwest');

title('Illustrative Received Power');

%% Simple first-tier co-channel reuse estimate

cellRadiusKm = 0.5;

reuseN = [1 3 7 12];

pathLossExponent = 3.5;

reuseDistance = sqrt(3 * reuseN) * cellRadiusKm;

% Approximate SIR calculation

sirLinear = (reuseDistance ./ cellRadiusKm).^pathLossExponent / 6;

sirDb = 10 * log10(sirLinear);

reuseTable = table( ...

reuseN(:), ...

reuseDistance(:), ...

sirDb(:), ...

'VariableNames', ...

{'ReuseClusterN', ...

'ApproxInterfererDistance_km', ...

'ApproxSIR_dB'});

disp('Co-channel Reuse Analysis');

disp(reuseTable);

%% SIR versus cluster size plot

figure;

plot(reuseN, sirDb, 'o-', 'LineWidth', 1.5);

grid on;

xlabel('Reuse Cluster Size N');

ylabel('Approximate SIR (dB)');

title('Simplified Co-Channel SIR versus Reuse Cluster Size');

%% Received power summary

fprintf('At %.2f km, estimated received powers are:\n', ...

distanceAtPointKm);

disp(table( ...

bandName, ...

rxAtPoint(:), ...

'VariableNames', ...

{'Band','Rx_dBm'}));

OUTPUT